MAISON DOCLAR

# Hardware, Radio and Pager Verification Protocol

Candidate equipment, regulatory gates, acceptance testing and custody

| Control | Value |
| --- | --- |
| Document ID | MD-USH-HWV-006 |
| Version | 1.0 |
| Status | DRAFT FOR CEO RATIFICATION |
| Doctrine | Every Guest, Considered. |
| Prepared | 04 September 2026 |

Authority: This document becomes Maison Doclar authority only after CEO ratification. Legal, radio-spectrum, equipment type approval, medical, security, accessibility, cultural and protocol determinations remain subject to the named competent authority. Vendor specifications are claims until verified.

# Contents

1. Candidate equipment register

2. Regulatory gate

3. Receipt inspection

4. Venue acceptance

5. Pager deployment decision

6. Procurement caveat

7. Acceptance certificate

# 1. Candidate equipment register

| Candidate | Advertised/user-supplied claim | Required verification |
| --- | --- | --- |
| BYHUBYENG CS-S1 call button | 433.92 MHz; claimed 80–100 m direct and >500 m open; claimed IPX67; >3,000 uses; 60×60×9 mm; adhesive/screw; indicative £3.36 | Candidate only: frequency/type approval, battery/serviceability, zone naming, false-call rate, range under crowd, mounting and ingress test |
| BYHUBYENG CS-200E watch pager | 433.92 MHz; claimed OLED/dot-matrix/touch; vibration and number; up to 200 buttons; 80 mAh; 72+ h standby; claimed IPX67; indicative £18.66 | Candidate only: pairing limits, simultaneous alerts, zone filtering, acknowledge/clear behaviour, actual runtime and visibility test |
| BYHUBYENG 160F repeater | 433.92 MHz FSK; claimed extension to 2,000 m; 5V/2A; 100–240V adapter; 129×71×26 mm; indicative £18.66 | Candidate only: confirm exact model/manual; repeater compatibility, duplication/loop behaviour, failover, UPS and placement test |
| Retevis RT22 / Baofeng BF-888S | Compact analogue UHF candidates; common accessory ecosystem | Do not call licence-free in Nigeria without NCC verification; exact programmed frequencies, power and type approval required |
| Motorola CP200d/R2 / Hytera HP5/HP6 | Commercial digital candidates for complex venues | Professional programming, lawful spectrum, interoperability, encryption/privacy decision, dealer support and total cost required |
| Clear acoustic tube 2-wire | Discreet separate earpiece and PTT | Supervisor/command candidate; hygiene, moisture, fit and acoustic safety |
| D-ring/C-hook or G-loop | Non-in-ear or secure loop styles | General/active roles; connector compatibility and sanitation |

# 2. Regulatory gate

Confirm Nigerian Communications Commission frequency use, equipment type approval and any licensing requirement for the exact radio and pager equipment—not only the product family.

Obtain supplier declaration, model/serial evidence, manuals and lawful programming record.

Do not program or operate business radios based on foreign “licence-free” claims.

Record venue permission and known local radio systems; coordinate with security/production frequency planning.

# 3. Receipt inspection

| Test | Method | Pass evidence |
| --- | --- | --- |
| Identity | Model, serial, firmware/manual and ordered specification | Photographic/custody record |
| Physical | Damage, band, button, seal, charger, connector | Inspection signed |
| Pairing | Each button/receiver/repeater programmed and uniquely labelled | Pairing matrix |
| Battery | Charge, timed load/standby sample, low-battery behaviour | Battery test log |
| Ingress | Manufacturer evidence plus non-destructive operational check | No false IP claim |
| Accessory | Earpiece/PTT connector, strain relief, hygiene parts | Role fit test |

# 4. Venue acceptance

Test before décor/crowd and again at realistic occupancy.

Press every deployed call button three times from installed position; confirm correct pager group, label, latency and no duplicate/ghost alert.

Test simultaneous calls, repeated presses, cleared calls, receiver restart, repeater power loss and substitute device.

Radio check every zone, stair, lift lobby, restroom approach, backstage, drop-off and command point.

Test primary and fallback channel, runner routes and emergency priority phrase.

Log dead spots; relocate/reduce scope rather than accepting uncertainty.

# 5. Pager deployment decision

| Class | Use |
| --- | --- |
| P0 | No guest buttons; proactive posts and rovers only |
| P1 | Discreet assistance points at remote/accessibility/VIP locations |
| P2 | One per selected seating cluster with defined service purpose |
| P3 | One per table only when visual, capacity, false-call and service tests pass |

# 6. Procurement caveat

Indicative prices are not approved budgets and may exclude shipping, duties, spares, chargers, programming, cases, labels, VAT/tax, batteries, dealer support and replacement. Purchase only after total landed cost and supportability review.

# 7. Acceptance certificate

| Gate | Status | Evidence/authority |
| --- | --- | --- |
| Exact model verified |  |  |
| NCC/type approval/spectrum decision |  |  |
| Battery and pairing |  |  |
| Crowd-range test |  |  |
| Power/UPS fallback |  |  |
| Hygiene/accessory fit |  |  |
| Spares and custody |  |  |
| Pilot acceptance |  |  |

# Detailed controlled operating annex

This annex expands the governing requirements into operational design, verification and evidence. It is normative only after ratification with the parent document.

## Frequency and equipment authority

433.92 MHz operation and business-radio use must be confirmed for the exact equipment, power, modulation and intended use. Foreign conformity marks or marketplace descriptions do not prove Nigerian authorisation.

Required practice. Retain supplier, type-approval, programming and serial records. Obtain competent advice where the regulatory position is unclear.

Acceptance evidence. Until verified, equipment may be bench-tested only within an authorised environment and is not declared operational standard.

## Range evidence

Open-space claims are not venue guarantees. Concrete, metal, LED systems, audio equipment, human bodies, placement and interference change performance.

Required practice. Test every installed transmitter at realistic occupancy, with doors in event position and equipment operating. Record latency, missed/duplicate calls and receiver group.

Acceptance evidence. A dead zone is corrected, excluded or covered by a human post; it is never accepted through optimistic wording.

## Battery management

Asset records show charge cycle, issue state, test time, expected duty, spare and return condition. Standby claims do not equal heavy-alert runtime.

Required practice. Charge on protected circuits; separate tested-ready, charging, quarantine and failed equipment.

Acceptance evidence. Low-battery behaviour is rehearsed. Replacement does not remove a post or lose unresolved alerts.

## Pager human factors

Labels must be readable and map unambiguously to physical zones. Alert patterns should distinguish routine assistance from priority staff signals if the equipment supports this.

Required practice. Test vibration visibility under movement, sleeves, noise and simultaneous calls. Train clearing and repeat-call handling.

Acceptance evidence. If every pager receives every call and alarm load becomes confusing, narrow deployment or use dedicated receiver groups rather than relying on attention.

## Radio acoustic safety and hygiene

Earpieces are assigned or sanitised under a documented method. Volume is set low enough for situational awareness and hearing safety.

Required practice. Route cables to avoid snagging and preserve professional appearance; strain-relief and connector compatibility are checked.

Acceptance evidence. A user who cannot hear clearly or develops discomfort changes accessory or role rather than increasing volume indefinitely.